MAINE PUBLIC DRINKING WATER

SOURCE WATER ASSESSMENT PROGRAM

BANGOR WATER DISTRICT

FLOODS POND WATERSHED

MARCH 2003

prepared for

Source Water Assessment Program

Drinking Water Program

Maine Department of Human Services

prepared by

Drumlin Environmental, LLC

Portland, Maine

MAINE PUBLIC DRINKING WATER

SOURCE WATER ASSESSMENT PROGRAM

BANGOR WATER DISTRICT

FLOODS POND WATERSHED

MARCH 2003

prepared for

Source Water Assessment Program

Drinking Water Program

Maine Department of Human Services

11 State House Station

Augusta, Maine  04333

prepared by

Drumlin Environmental, LLC

15 Franklin Street, P. O. Box 392

Portland, Maine 04112-0392

(207) 771-5546

MAINE PUBLIC DRINKING WATER

SOURCE WATER ASSESSMENT PROGRAM

BANGOR WATER DISTRICT

FLOODS POND WATERSHED

TABLE OF CONTENTS

Section      Title                                                                                                            Page No.

|  | EXECUTIVE SUMMARY | ii |
| --- | --- | --- |
| 1.0 | INTRODUCTION | 1 |
| 2.0 | WATER SUPPLY SOURCE | 1 |
|  | 2.1 Description | 1 |
|  | 2.2 Land Use | 3 |
| 3.0 | INVENTORY OF POTENTIAL WATER QUALITY CONCERNS | 3 |
|  | 3.1 Background | 3 |
|  | 3.2 Source Sensitivity to Land Use | 5 |
| 4.0 | # ASSESSMENT OF FLOODS POND | 5 |
|  | 4.1 Watershed | 5 |
|  | 4.2 Shoreland | 6 |
|  | 4.3 Intake | 6 |
| 5.0 | SWAP RANKING AND RECOMMENDATIONS | 7 |
|  | 5.1 Ranking of Susceptibility | 7 |
|  | 5.2 Recommendations | 8 |
|  | Table 1 - Floods Pond Supply | 2 |
|  | Table 2 - Sources of SWAP Information | 4 |
|  | Table 3 - Floods Pond Surface Water Assessment | 7 |
|  | Figure 1 - Public Water Supply and Surrounding Watershed | End |
|  | Figure 2 - Land Under Protective Control | End |
|  | Figure 3 - Multi-Resolution Land Characterization | End |
|  | Figure 4 - Elevation Contours | End |
|  | Appendix A - SWAP Assessment Guidelines | End |

# BANGOR WATER DISTRICT

# FLOODS POND WATERSHED

# EXECUTIVE SUMMARY

The Bangor Water District has 10,000 service connections serving an overall population of approximately 55,000 from Floods Pond.  The 5,602-acre watershed of Floods Pond and nearby Burnt Pond is located in Otis, Dedham and Clifton.  The surface area of Floods Pond is of moderate size.  The average depth of the pond is shallow to moderate with one area in excess of 100 feet deep.  Nearly the entire watershed area is covered by forest growth.  The shoreland and upland watershed areas are owned by the Water District or protected through conservation easement and landowner agreements.  Using locked gates, posted signs, public awareness education and visual observation, the Water District maintains a high level of control to prohibit access and recreational activities in the watershed.

No significant land use threats were identified during the reconnaissance of the watershed.  The Water District oversees the forest through a management plan that is protective of the water resource.  In addition, watershed protective measures preclude agricultural activities including grazing animals and other farming activities as a multi-barrier approach to eliminate the introduction of Giardia and Cryptosporidium into the water supply.

The Water District operates a pump station and treatment plant in the watershed of Floods Pond to draw water from the intakes and deliver it into the distribution system.  The water is used as an unfiltered source.  Treatment includes prechlorination, ozonation, chloramination, fluoridation and lime addition for pH adjustment.  Raw water monitoring of the pond for several parameters and testing after treatment show good water quality from this source, i.e., all test parameters meet applicable state and federal drinking water standards.

Based on the undeveloped, forested nature of the watershed, extensive ownership and protection controls by the Water District, recreational controls and public awareness, the overall susceptibility of the Floods Pond water supply is considered to be low.

MAINE PUBLIC DRINKING WATER

SOURCE WATER ASSESSMENT PROGRAM

BANGOR WATER DISTRICT

FLOODS POND WATERSHED

1.0    INTRODUCTION

The operation of public water systems in Maine is governed by the federal Safe Drinking Water Act (SDWA), which was first passed in the 1970s and later amended in 1986 and 1996.   The federal government delegated authority for enforcing the SDWA to the state under the Maine Department of Human Services (DHS) Drinking Water Program (DWP).   In 1998, the Source Water Protection Section was established in the DWP to implement the Maine Public Drinking Water Source Water Assessment Program (SWAP).  The SWAP process is being conducted through a cooperative effort between the DWP staff, Bangor Water District (Water District) staff and Drumlin Environmental, LLC (Drumlin), the environmental consulting firm contracted through DHS to provide technical assistance for the project.

The purpose of the SWAP evaluation for the Water District is to assess the susceptibility of its Floods Pond drinking water supply (PWSID  90110) to potential threats of contamination.   The long-term goal is to protect the water supply source.   To do this, the SWAP process has compiled information to assist the Water District in future planning that will help to control potential threats to the water quality of Floods Pond.

This Report describes the SWAP for the Bangor Water District and is organized as follows:

- Section 2.0  summarizes the physical characteristics of the water supply at Floods Pond;

- Section 3.0  describes the variety of conditions and activities that could pose a threat to water quality;

- Section 4.0  provides an assessment of the threats within the Floods Pond watershed; and,

- Section 5.0  ranks the susceptibility of the water quality at Floods Pond and provides recommendations for future protection of water quality.

2.0   WATER SUPPLY SOURCE

2.1   Description

The watershed of Floods Pond is located primarily in Otis, Maine; however, the watershed extends into the Town of Clifton to the north and the Town of Dedham to the west.  The Water District considers the drainage area of Floods Pond to include other nearby water bodies, which drain into Floods Pond.  This broader watershed delineation includes Burnt Pond, located immediately to the west with a surface area of 326 acres, Little Burnt Pond at 10 acres and Buggy Pond at five acres.  The ponds and watershed areas are depicted in Figure 1.  The components and description of the Water District source are presented below in Table 1.

# TABLE 1

# FLOODS POND SUPPLY

| # Category | # Description |
| --- | --- |
| Water System Name: | Bangor Water District |
| Surface Water Source(s): | Floods Pond |
| Water System Type: | Non-transient, community system |
| Watershed Location: | Otis, Clifton and Dedham, Maine |
| Source Surface Water Area: | 654 Acres |
| Source Watershed Area: | 3,439 Acres (Floods Pond) 5,602 Acres (Floods & Burnt Ponds combined) |
| Average/Maximum Depths (estimated): | 45 ft; 140 ft |
| Water Volume in Storage: | 9.5 billion gallons |
| Population Served (customers): | Services: 10,000 Population: 55,000 (serving Bangor & portions of Clifton, Eddington, Veazie, Hermon, Hampden and Orrington) |
| Type of Treatment: | Prechlorination, ozone treatment, secondary disinfection with chloramines (chlorine and ammonia), fluoridation, lime addition for pH adjustment. |
| Filtration: | Filtration waiver in effect |
| Estimated Daily Water Use: | 4.8 to 5.1 MGD average (10 MGD peak use) |

The Water District has two intake pipes extending into the pond.  One intake is located approximately 185 feet from the Johnston Pump Station, which is set back about 20 feet from the shoreline.   The second intake pipe is located 302 feet from the pump station.  The intakes and pump station are located on a small peninsula of land located at the north end of Floods Pond.  The intakes are 36-inch diameter reinforced concrete pipe equipped with bar screens at the end of each pipe.  The intake pipes rest on a stone base built on the bottom of the pond.  The intake screens are at elevations 292.0 and 282.5 feet above mean sea level, respectively.  Normal water level (Elevation 305.5 ft, msl) is 13.5 feet of water over the top of the screen at the intake located closer to shore and 22.5 feet of water over the screen at the intake located farther from shore.

Water flows by gravity from the intakes into a 35-foot deep intake well located in the pump station.  The water flows through a second wire mesh stainless steel screen prior to entering the clear well.  Prechlorination occurs in the intake well to prevent growth of bacteria.  The water is pumped through a transmission main to the Butler Ozone Treatment Facility located about 4,000 feet to the north along the access road.  The treatment plant was built in 1995.  The pump station located at the pond was built earlier in 1958-1959.   The water entering the plant is treated with ozone, chloramines and fluoride and is pH-adjusted using lime.  The water subsequently enters a surge tank and then flows by gravity into a 30-inch diameter transmission main that feeds into the distribution system.  The distribution system infrastructure includes seven standpipes, which store 15.8 million gallons of water.

Floods Pond has been estimated to store 9.5 billion gallons of water, of which approximately four billion gallons are located below the intakes.  The pond has a calculated safe yield of 8.3 million gallons per day.   The volume of water held in the pond is estimated to be completely replaced by flushing on average every 2 years and 11 months.   The pond has a dam located at its southern end.  The dam has a fixed spillway.   The Water District monitors the dam as part of its watershed inspection.

2.2   Land Use

The Water District owns or protects 98 percent of the land in the watershed.  The protections include land under conservation easement and landowner agreements.  The ownership extends throughout the watershed with only a few parcels remaining in private ownership.  Figure 2 shows the land under permanent control in the watershed.   The private land areas

are at the watershed boundary located to the north in Clifton and to the south near the pond outlet, both of which are remote from the intake area.

The Water District completed acquisition of a 200-foot wide buffer of land around the entire perimeter of Floods Pond and Burnt Pond in 1958.   The ownership provides complete control of the pond shoreland.  Other upland properties were purchased beginning the summer of 1993.

Eight roads (seven graveled) provide access into the watershed.  Only one of these roads is an all-seasons road.  These roads are owned by the Water District and have locked gates and signs to prevent public access.  The watershed is undeveloped except for the caretaker’s house located near the pump station, and the water treatment plant.  Two hunting camps are located in the drainage area of Burnt Pond.  Approximately 98 percent of the watershed is evergreen, deciduous and mixed forest growth.  The Water District has a forest management plan, which provides for selective cutting and the use of best management practices.  The remaining area includes some wetlands, roadways and transitional areas of growth.   A map of the land use classifications for the watershed and surrounding area is shown in Figure 3.

Floods Pond and other ponds in the watershed are closed to recreational use.  Recreation is prohibited through state law and Water District ownership.  Signs are posted at roadway and trail access points to prohibit public use.  The Maine Department of Inland Fisheries and Wildlife has established a program to protect landlocked arctic charr fisheries in Floods Pond.  Fishing, boating and swimming are not allowed on Floods Pond.

3.0   INVENTORY OF POTENTIAL WATER QUALITY CONCERNS

### 3.1   Background

### The Source Water Assessment Program (SWAP) is intended to assist water companies and departments in protecting the quality of their surface water supply by identifying potential threats to water quality.  Two factors have been considered in identifying and assessing the potential risk posed by a threat: (a) location in relation to the intake and (b) the nature of a potential threat.

This SWAP assessment looks for potential threats to water quality in three zones:

- The intake zone within 1,000 feet of the intake;

- The shoreline zone, within 250 feet of the shoreline of the surface water body; and,

- The watershed zone, extending to the limits of the surface watershed.

Activities in each of these zones have a different potential effect on the quality of water in the surface supply.

Within each of the three zones, the SWAP assessment examined a variety of conditions, land uses and practices that have a potential to influence water quality.  These features generally fall into one of the following categories:

- Physical characteristics of the watershed, for example the presence of wetlands, steep topography or erodible soils;

- General land uses and development patterns, for example the percentage of developed versus undeveloped land and controls to guide future development; and,

- Specific activities that involve chemical handling or may release pathogens (e.g., fecal coliform) with the potential to degrade water quality.

The SWAP assessment also considered available water quality data from the surface supply as an indicator of existing conditions.  Appendix A describes specific guidance used during the SWAP to inventory potential factors influencing water quality.  Appendix A also lists specific activities included in the SWAP assessment.  Table 2 lists general sources of information consulted during SWAP research.

# TABLE 2

SOURCES OF SWAP INFORMATION

| Information Type | Information Source |
| --- | --- |
| Watershed Characteristics | US Geological Survey (Topography, Hydrology), Soil Conservation Service (Soils), Maine Geological Survey (Geology) |
| Raw Water Quality | Water District Monitoring, PEARL Database |
| Potential Contamination Sources | DEP Databases and Water District |
| Land Use and Zoning | Water District and Municipal Records |
| Other Analyses | Other DEP Databases |

3.2  Source Sensitivity to Land Use

The key watershed features that define the sensitivity of the water supply source to land uses are briefly described below:

- Natural Features:   The watershed is entirely forested.   The natural, undeveloped watershed conditions are protective of the water supply source.

- Man-made Features:   The primary buildings and roadways located in the watershed are under the control of the Water District, which manages activities to be protective of the source.

- Specific Activities:   Access to the pond shoreland is restricted.  Recreational activities are not permitted.

4.0   ASSESSMENT OF FLOODS POND

Floods Pond is used as an unfiltered, ozonated and chloraminated water supply source.   Historically, the water quality has been very good.   Water test records (i.e., TE2 & TE3) after treatment show all parameters are within acceptable standards for public drinking water.  The water source has a “synthetic organics waiver” which provides for a three-year exemption from certain testing requirements (i.e., pesticides, herbicides, fungicides and organic chemicals).

Raw water is tested for turbidity, fecal coliform, total coliform and E. Coli bacteria at the Johnston pump station.  Turbidity, measured at two-minute intervals, shows an average reading of 0.35 NTU with peak values up to 0.8 NTU.   Fecal coliform is tested five times per week.  On average, these tests show under 5 colonies/100 mls with occasional peaks at 15 colonies/100 mls.

Water is also tested at 18 sampling sites located throughout the watershed.  The sampling begins after the ice thaws in the spring and continues into the fall (i.e., April through September).  Monthly testing at the 18 sampling sites includes turbidity, total organic carbon, total coliform and E. Coli bacteria.   Dissolved oxygen and temperature vertical profile measurements, chlorophyll-a, total phosphorus and secchi disk readings are taken monthly at the deepest location in the pond.   Collectively, the data show low concentrations of dissolved constituents and low biological activity.   No problems with algae growth have been observed in the pond.

4.1   Watershed

Floods Pond and its watershed comprise a large drainage area.  There are 16 tributary streams flowing into the pond with only one that flows throughout the year.  The depth and area of the pond provides for a sizeable storage volume as stated previously.   The watershed is almost entirely forested with man-made development limited primarily to the operations of the Water District.  Timber management and harvesting is conducted in the watershed under the control of the Water District.  The forest is selectively harvested and cutting operations are required to keep a setback distance of 200 feet from the ponds.   Chemical spraying is not permitted.

Most of the upland slopes are steep and drain toward Burnt and Floods Ponds.  In the event of a significant loss of trees due to forest fire, the slopes would become exposed to rainfall and increased runoff.   The added runoff could potentially increase the transport of eroded soils, dissolved solids and constituents derived from the burning of organic material.  Under these conditions, the pond would be more vulnerable to drainage in the burned portion of the watershed.

The watershed includes an area of 5,602 acres.   Only 141 acres in the watershed is privately-owned.  The protected lands include 3,669 acres owned by the Water District, 700 acres in conservation easement, 102 acres in protective landowner agreements and 990 acres of surface water bodies under state control.   Access roads leading into the watershed have locked gates, which are controlled by the Water District.   Signs are posted to prohibit access into the watershed, particularly on trails accessible to recreational vehicles.

Review of the DEP databases of regulated activities and observations made during our reconnaissance of the watershed found no significant point or non-point sources of potential contamination that would be a concern to the water supply.

The Water District is responsible for the access road maintenance, treatment plant operations, Johnston Pump Station and caretaker’s residence.   Chemicals and fuels are used in the operations of the water system.   Secondary containment is provided for these materials, which are held in bulk storage tanks.  The Water District has established safe practices for materials handling in order to avoid an accidental spill or release of treatment chemicals or petroleum.

4.2   Shoreland

Other than the Water District facilities, the entire shoreland of Floods Pond and the other smaller water bodies located in the watershed are undeveloped.  The shoreland is protected in a natural state by the ownership and control of the Water District.

The shoreland topography around the pond varies from gentle to steep slopes (see Figure 4).  Steep slopes occur along a majority of the eastern and western shores of the pond although the forested cover helps to stabilize the slopes.   If the forest is removed or damaged by fire, natural cover would be lost and soils would be exposed to increased runoff.

4.3   Intake

The water supply intakes are located in the pond within a few hundred feet from shore.   The intake area is completely controlled by the Water District; therefore, unauthorized access is not likely to occur.   As stated previously, materials (e.g., chemicals and fuel) are handled at the pump station as part of the water system operation.  The Water District has established safe procedures for the delivery and storage of these materials and will continue to review these practices for possible improvements.

Lakes and ponds normally experience an aging process known as eutrophication, which is caused by various natural and man-made influences.   Phosphorus from runoff, fertilizers, sewage and other sources is a primary factor affecting pond eutrophication since phosphorus nourishes plant and algae growth in the pond.   The amount of algae productivity in the pond is characterized in terms of three Trophic States: (1) Oligotrophic = low, (2) Mesotrophic = moderate, and (3) Eutrophic = high.  The Trophic State can be identified based on chlorophyll and phosphorus content and by secchi disk transparency data, which measures the clarity of the pond water.  The results of Water District monitoring in Floods Pond shows the pond to be in an Oligotrophic state, i.e., low biological productivity.  In the last ten years, secchi disk measurements averaged 20 to 23 feet below water surface.  Total phosphorus has typically been less than 5 parts per billion (ppb) with occasional peaks to 10 ppb.   Dissolved oxygen has averaged in the range from 8 to 10 parts per million.  Chlorophyll-a has averaged 2 ppb from the surface to 33 feet deep.  Color is generally less than 5 units.  Turbidity measurements in the pond are usually less than 0.5 NTU.

According to a surface water classification database developed by the DEP, the Trophic State of Floods Pond is listed as mesotrophic.  This suggests an inconsistency with the information presented above and may warrant further discussion with the DEP.  The pond water is ranked as fully attaining the GPA classification with respect to primary contact, aquatic life and trophic stability criteria.

Based on all the testing completed to date on the raw water from Floods Pond, the water is a high quality source.

5.0   SWAP RANKING AND RECOMMENDATIONS

5.1   Ranking of Susceptibility

The SWAP assessment factors indicate that overall susceptibility of the water quality in Floods Pond is low.  This conclusion is based on the protections that exist for 98 percent of the watershed, the forested land use and the absence of potential threats from land use activities.  Specific factors considered in assessing the overall risk are summarized in Table 3.

# TABLE  3

# FLOODS POND SURFACE WATER ASSESSMENT

| Zone | Measure | Findings | Risk Level |
| --- | --- | --- | --- |
| Watershed | Ambient Water Quality Existing Conditions Future Development Overall | Class GPA, in full compliance. Land throughout a majority of the watershed is protected; forest cutting managed to be protective; public use is prohibited. Water District ownership and protective agreements will prevent future development in a majority of the watershed. | Low Low Low Low |
| Shoreland | Pond Classification Soils Activities Posing a Threat Potential for Future Threats Overall | Oligotrophic Steeply sloping soils are covered by natural forest growth although erosion potential is high, if disturbed. None identified. None identified. | Low Low Low Low Low |
| Intake | Raw Water Quality Ownership/Control Activities Posing a Threat Potential for Future Threats Overall | Testing shows good quality. Water District ownership protection around intake area. Access to intake area is not allowed without Water District permission. None identified. | Low Low Low Low Low |
| Overall |  |  | Low |

5.2   Recommendations

The overall ranking for the pond susceptibility to threats of contamination is low.  The Water District has established protection for a majority of the watershed land area.  The Water District controls public access and prohibits recreational activities using signs and through visual observation and monitoring.  The quality of water in the pond is monitored through an extensive testing program.  The forest is managed through a plan that promotes harvesting practices that are protective of water resources.  The Water District is active in maintaining strong public awareness for water supply protection in the surrounding communities.  The Water District can continue its efforts with watershed protection through the following activities.

- Continue to implement a thorough water quality-monitoring program to look for potential trends over time.

- Continue working with landowners to establish protection on the land area that remains in private ownership.

- Ensure that materials handling procedures are reviewed and updated, as needed, with changes in personnel or changes with vendor deliveries.

APPENDIX A

#### SWAP ASSESSMENT GUIDELINES

MAINE PUBLIC DRINKING WATER

SOURCE WATER ASSESSMENT PROGRAM

BANGOR WATER DISTRICT

FLOODS POND WATERSHED

# APPENDIX A

# ASSESSMENT GUIDELINES

| Assessment Item/Threat | Low | Moderate | Significant |
| --- | --- | --- | --- |
| Physical Characteristics of the Watershed Watershed boundary, area Area of water body Tributaries Watershed topography Wetlands Water body depth Sand and gravel aquifers Soil types (erodibility and slope) | smaller smaller few low many deep few low | >> >> >> moderate >> >> some moderate | larger larger many steep, rugged few shallow many high |
| Raw Water Quality Secchi Disk Transparency Turbidity pH Phosphorus Pesticides and herbicides Color Total Coliform and E. Coli Dissolved Oxygen Temperature VOCs Chlorophyll-a Total Organic Carbon | > 8 M < 1 NTU 6.5-8.5 < 10 ppb ND < 5 > 29/100 ml >7 mg/L ambient absent < 2 ppb < 4 mg/L | 4-8 M < 5 NTU N/A < 20 ppb N/A 5-15 occasional 5-7 mg/L N/A 2-6 ppb 4-8 mg/L | < 4 M > 5 NTU < 6.5 or > 8.5 > 30 ppb detected > 15 > 142/100 ml < 5 mg/L > 10o C ambient detected > 6 ppb > 8 mg/L |
| Potential Contamination Source Inventory UST/AST leak sites Municipal/Comm/Special Landfill Compost sites Demolition debris sites Ash/septage storage/utilization Superfund site Sand/salt storage sites Petroleum spill sites Hazardous waste sites Wastewater treatment facilities Food residuals utilization sites Uncontrolled hazardous sites Tank farms Industrial complexes | Absent from watershed or actively monitored and controlled or fully remediated. | Present in watershed, some operational deficiencies and/or remediation is underway. | Present and unmonitored and/or unremediated. |

| Assessment Item/Threat | Low | Moderate | Significant |
| --- | --- | --- | --- |
| Potential Contamination Source Inventory Non-point pollution sources Transfer stations Sludge utilization sites Automobile graveyards Engineered subsurface systems Woodyards Underground injection wells Surface impoundments Mining/mineral processing sites Overboard discharges to rivers Active UST sites Roadways Railroads Utility Rights-of-way Boat launches Commercial farms CAFOs Commercial forest operations Private septic systems Home heating oil tanks Airports Bathing beaches Combined sewage overflows | If present, at low density, properly installed and operated. In compliance with applicable local, state and federal regulations. | >>>> | High density, little evidence of active management, documented incidence of problems and failures. |
| Land Uses and Zoning Residential density Percent of watershed Owned by PWS Public lands or conservation Forested With protective zoning Agricultural Impervious cover Industrial/commercial Waterbody uses: Restricted areas Ice fishing Boating Swimming Recreational vehicles | > 2 Ac/lot total > 80% for all protected uses < 20 % total < 20 % total < 20 % total large, posted limited to absent | 1-2 Ac/lot intake and shoreland protected, others developed 20-30% total 20-30% total 20-30% total intake zone moderate, well-managed | < 1 Ac/lot critical areas (intake, shoreland) unprotected or developed > 30% total of 3 > 30% total of 3 > 30% total of 3 small to none prevalent, uncontrolled |
| Other Analyses Trophic State Index Vulnerability Index Build-out Analysis | < 25 low fully developed | 25-60 moderate >>> | > 60 high high potential for development |