MAINE PUBLIC DRINKING WATER

#### SOURCE WATER ASSESSMENT PROGRAM

MADISON AND ANSON WATER DISTRICTS

HANCOCK POND WATERSHED

MARCH 2003

prepared for

Source Water Assessment Program

Drinking Water Program

Maine Department of Human Services

prepared by

Drumlin Environmental, LLC

###### Portland, Maine

MAINE PUBLIC DRINKING WATER

SOURCE WATER ASSESSMENT PROGRAM

MADISON AND ANSON WATER DISTRICTS

HANCOCK POND WATERSHED

MARCH 2003

prepared for

Source Water Assessment Program

Drinking Water Program

Maine Department of Human Services

11 State House Station

Augusta, Maine  04333

prepared by

Drumlin Environmental, LLC

15 Franklin Street, P. O. Box 392

Portland, Maine 04112-0392

(207) 771-5546

MAINE PUBLIC DRINKING WATER

SOURCE WATER ASSESSMENT PROGRAM

MADISON AND ANSON WATER DISTRICTS

HANCOCK POND WATERSHED

TABLE OF CONTENTS

Section      Title                                                                                                            Page No.

|  | EXECUTIVE SUMMARY | ii |
| --- | --- | --- |
| 1.0 | INTRODUCTION | 1 |
| 2.0 | WATER SUPPLY SOURCE | 1 |
|  | 2.1 Description | 1 |
|  | 2.2 Land Use | 3 |
| 3.0 | INVENTORY OF POTENTIAL WATER QUALITY CONCERNS | 4 |
|  | 3.1 Background | 4 |
|  | 3.2 Source Sensitivity to Land Use | 5 |
| 4.0 | # ASSESSMENT OF HANCOCK POND | 5 |
|  | 4.1 Watershed | 6 |
|  | 4.2 Shoreland | 6 |
|  | 4.3 Intake | 7 |
| 5.0 | SWAP RANKING AND RECOMMENDATIONS | 8 |
|  | 5.1 Ranking of Susceptibility | 8 |
|  | 5.2 Recommendations | 9 |
|  | Table 1 - Hancock Pond Supply | 2 |
|  | Table 2 - Sources of SWAP Information | 5 |
|  | Table 3 - Hancock Pond Surface Water Assessment | 8 |
|  | Figure 1 - Public Water Supply and Surrounding Watershed | End |
|  | Figure 2 - Multi-Resolution Land Characterization | End |
|  | Figure 3 - Potential Erodible Soils | End |
|  | Appendix A - SWAP Assessment Guidelines | End |

# MADISON AND ANSON WATER DISTRICTS

# HANCOCK POND WATERSHED

# EXECUTIVE SUMMARY

The Madison and Anson Water Districts operate a combined water supply system that serves more than 4,000 people from Hancock Pond.  The 3,095-acre watershed of Hancock Pond is located in the towns of Embden and New Portland, and in Lexington Township.  Approximately 94 percent of the watershed is in forest growth.  The northern and western upland watershed areas are sparsely developed with residential homes.  The western half of the pond shoreland is heavily developed with camps and seasonal homes.  The eastern half of the pond shoreland is undeveloped and protected by local zoning.  The roads that provide access to the dam are unimproved and difficult to pass with a vehicle; however, the land is privately owned and generally unrestricted to public access.   Additional safeguards for shoreland use in Embden are provided through the 250-foot Shoreland and Resource Protection Zones established around the pond and an ongoing program to upgrade subsurface wastewater disposal systems.

No significant land use threats were identified during the reconnaissance of the watershed.  The principal activities to monitor in the watershed include timber harvesting, camp road development/maintenance and unrestricted recreational use.  Recreational pressure may increase if interest by the state to construct a new public boat launch site is pursued.  The Water Districts have installed a sign at the pond outlet to identify the water supply source to the public.  The Water Districts maintain and control the dam at the pond outlet.  Additional signs, buoys, markers, etc. would be helpful to increase public awareness of the need to protect the pond and intake area.  Land use provisions or strategies to minimize recreational threats to the source and intake area should be considered.

The Water Districts’ intake, located in the reservoir in approximately 15 feet of water at a distance of 240 feet from shore, delivers water by gravity through a pipeline to a sand filter treatment plant located to the south of the source.  Raw water monitoring for routine parameters, and testing for TE2, TE3 and other parameters have indicated good water quality from this source, i.e., showing all test parameters in compliance with public drinking water standards.   The withdrawal of water from the pond may reach or exceed the safe yield of the watershed under natural, low-flow conditions; therefore, the Water Districts should consider evaluating this condition to see if any further actions are necessary.

The natural and predominantly undeveloped conditions of the watershed are protective of the source.   However, recreational access and activities on the pond are unrestricted and protection strategies are not established to mitigate development impacts in the upland watershed areas.  In addition, the Water Districts’ control of land use in another municipality’s jurisdiction is minimal.  There is a potential for increased recreational pressures and for exceeding the safe yield of the watershed.   These factors result in an overall susceptibility ranking of low to moderate for the Hancock Pond supply.

MAINE PUBLIC DRINKING WATER

SOURCE WATER ASSESSMENT PROGRAM

MADISON AND ANSON WATER DISTRICTS

HANCOCK POND WATERSHED

1.0    INTRODUCTION

The operation of public water systems in Maine is governed by the federal Safe Drinking Water Act (SDWA), which was first passed in the 1970s and later amended in 1986 and 1996.   The federal government delegated authority for enforcing the SDWA to the state under the Maine Department of Human Services (DHS) Drinking Water Program (DWP).   In 1998, the Source Water Protection Section was established in the DWP to implement the Maine Public Drinking Water Source Water Assessment Program (SWAP).  The SWAP process is being conducted through a cooperative effort between the DWP staff, Madison and Anson Water Districts’ (Water Districts) staff and Drumlin Environmental, LLC (Drumlin), the environmental consulting firm contracted through DHS to provide technical assistance for the project.

The purpose of the SWAP evaluation for the Water Districts is to assess the susceptibility of the Hancock Pond drinking water supply (PWSID  90930 & 90040) to potential threats of contamination.   The long-term goal is to protect the water supply source.   To do this, the SWAP process has compiled information to assist the Water Districts in future planning that will help to control potential threats to the water quality of Hancock Pond.

This Report describes the SWAP for the Madison and Anson Water Districts and is organized as follows:

- Section 2.0  summarizes the physical characteristics of the water supply at Hancock Pond;

- Section 3.0  describes the variety of conditions and activities that could pose a threat to water quality;

- Section 4.0  provides an assessment of the threats within the Hancock Pond watershed; and,

- Section 5.0  ranks the susceptibility of the water quality at Hancock Pond and provides recommendations for future protection of water quality.

2.0   WATER SUPPLY SOURCE

2.1   Description

The Hancock Pond watershed is located primarily in the Town of Embden, Maine with more than half of the watershed area extending further north into Lexington Township.  A small area of the watershed is also located in the Town of New Portland.  The pond and watershed area are depicted in Figure 1.  The components and description of the water supply   source are presented below in Table 1.

# TABLE 1

# HANCOCK POND SUPPLY

| # Category | # Description |
| --- | --- |
| Water System Name: | Madison and Anson Water Districts |
| Surface Water Source(s): | Hancock Pond |
| Water System Type: | Non-transient, community system |
| Watershed Location: | Embden, New Portland and Lexington TWP, Maine |
| Source Surface Water Area: | 314 Acres |
| Source Watershed Area: | 3,095 Acres |
| Average/Maximum Depths (estimated): | West End: 10 & 16 ft; East End: 35 & 105 ft |
| Water Volume in Storage: | Undetermined; safe yield est. in 1993 at 0.5 MGD |
| Population Served (customers): | Services: Anson-650, Madison-1,000, & Embden-100 Population: Est. 4,000+ |
| Type of Treatment: | Chlorination, Sodium Fluoride & Sodium Silicate for corrosion control |
| Filtration: | Slow sand filtration system in use |
| Estimated Daily Water Use: | 600,000 GPD average; 1 MGD peak use |

The water supply intake is positioned in the pond approximately 240 feet from shore, at the southeast end of the pond, near the outlet into Hancock Stream.  The pond level and flow out of the pond into the stream are controlled by a concrete dam originally built in 1923 and rebuilt in 1963.  The dam is about 163 feet long and four feet wide along its base.  The dam height is six feet and includes a 12-foot wide spillway.  The water supply intake consists of a 24-inch diameter stainless steel screen that rests on the pond bottom in about 15 feet of water.  The screen is equipped with an airline to periodically blow out and clean the screen openings.  The intake is not marked with buoys or signs to indicate its exact location.

Water flows by gravity from the intake through a 16-inch diameter transmission main to the filtration plant, which was built in 1994 with a design capacity of 1.2 million gallons per day.  The plant is located approximately 8,000 feet to the southeast and downstream from the pond and is accessed from the West Shore Road.  The plant operations are automated although certain flow controls are operated manually in response to the demand required by the users on the distribution system.  The raw water enters the plant and, if needed, is diverted through a microscreen during high turbidity flows.  The water is initially filtered through the slow sand filters and subsequently treated with chlorine for disinfection.   Sodium fluoride is added to aid user dental care and sodium silicate is added for pH adjustment/corrosion control.  The water flows into a clear well to increase the contact time before entering the distribution system.

The Hancock Pond is used as a shared water supply for the towns of Anson and Madison and for a small number of residents in Embden.  The water system is overseen by a joint board comprised of three members each from Madison and Anson.  The costs to operate the water system are shared by Anson and Madison based on the total volume of water used in each community.  The residents served in Embden live along the West Shore Road and Embden Pond Road, which are located near the upstream end of the transmission main.  The transmission main carries water over a distance of approximately 12 miles to the central business area of Madison and Anson.  At the center near the bridge, the water becomes separated into the two distribution systems that are operated independently by each Water District.  Anson is also responsible for operating and maintaining the North Anson distribution system.

The data obtained during the SWAP interview from the Water Districts indicated that average use for the entire system is approximately 600,000 GPD.  The 1993 safe yield calculated for the pond was determined to be 500,000 GPD.  These figures suggest that the amount of water being removed from Hancock Pond (on the average) may be at or above the upper limit of the available capacity of the watershed.   It was reported that in the summer and especially during dry periods, little or no flow drains from the pond outlet.   Pond level fluctuations are reported to have varied in the past from a few inches to as much as 14 inches.   Further study may be warranted to re-evaluate the current use and future demand anticipated on safe yield of this source and to assess the need for supplementing the water supply from other sources.

2.2   Land Use

The Hancock Pond watershed is located outside the towns of Anson and Madison.  Municipal control of the land area is under the jurisdiction of the town of Embden and Lexington Township. Water District control is limited to the dam located at the outlet of the pond.  Most of the watershed area located in Lexington Township is owned as timberland by a paper company.   There is no significant land area in the watershed under permanent protective ownership.

Embden has adopted a 250-ft Shoreland Zone around the pond.  Embden has also adopted an ordinance that requires certification of subsurface wastewater disposal systems so that they comply with the current sanitary code.  The town has sent a letter to each camp owner to provide notice of the need to comply with this new ordinance.  As a result, several sites have already been upgraded with new septic disposal systems that meet the certification requirements.   Systems that are not certified must either be replaced with a certified system or appropriate documentation must be provided to demonstrate compliance.

The western half of the Hancock Pond shoreline is heavily developed with camps, seasonal and year-round homes.  In several locations, the developed sites have encroached on or close to the pond shore.  For example, at the western end of the pond is a large, warehouse-type building setback from the pond that has reportedly been used to store a floatplane.

The eastern half is almost entirely undeveloped and protected by a 250-ft Resource Protection Zone where no building is allowed.   Initial contacts with town representatives indicated an interest by the state to have a public boat launch site on the north shore of the pond.  The shore would be accessed from the existing road located in Lexington TWP.

The upland watershed beyond the shore of the pond is sparsely developed with only a few residential homes and limited roadways located primarily in the north-central and western portions of the watershed.  A new subdivision development is anticipated to be developed on the western side of the watershed approximately at the midpoint of the pond.  Approximately 94 percent of the watershed is evergreen, deciduous and mixed forest growth.  Wetlands, grasses, shrubland and a minor presence of croplands comprise most of the remaining area.   A map of the land use classifications for the watershed and surrounding area is shown in Figure 2.

There are no limits to public access for recreational activities on the pond.   Access into the intake area is unrestricted although the roads are rough and difficult to pass except for four-wheel drive vehicles.   A large sign has been posted on shore at the outlet to indicate use of the pond as a water supply source.   All types of water recreation, i.e., swimming, fishing, motorboats, etc., are allowed on the pond.  As stated above, additional recreational use is likely to occur if public access is developed to the pond.

3.0   INVENTORY OF POTENTIAL WATER QUALITY CONCERNS

### 3.1   Background

### The Source Water Assessment Program (SWAP) is intended to assist water companies and Districts in protecting the quality of their surface water supply by identifying potential threats to water quality.  Two factors have been considered in identifying and assessing the potential risk posed by a threat: (a) location in relation to the intake and (b) the nature of a potential threat.

This SWAP assessment looks for potential threats to water quality in three zones:

- The intake zone within 1,000 feet of the intake;

- The shoreline zone, within 250 feet of the shoreline of the surface water body; and,

- The watershed zone, extending to the limits of the surface watershed.

Activities in each of these zones have a different potential effect on the quality of water in the surface supply.

Within each of the three zones, the SWAP assessment examined a variety of conditions, land uses and practices that have a potential to influence water quality.  These features generally fall into one of the following categories:

- Physical characteristics of the watershed, for example the presence of wetlands, steep topography or erodible soils;

- General land uses and development patterns, for example the percentage of developed versus undeveloped land and controls to guide future development; and,

- Specific activities that involve chemical handling or may release pathogens (e.g., fecal coliform) with the potential to degrade water quality.

The SWAP assessment also considered available water quality data from the surface supply as an indicator of existing conditions.  Appendix A describes specific guidance used during the SWAP to inventory potential factors influencing water quality.  Appendix A also lists specific activities included in the SWAP assessment.  Table 2 lists general sources of information consulted during SWAP research.

# TABLE  2

SOURCES OF SWAP INFORMATION

| Information Type | Information Source |
| --- | --- |
| Watershed Characteristics | US Geological Survey (Topography, Hydrology), Soil Conservation Service (Soils), Maine Geological Survey (Geology) |
| Raw Water Quality | Water District Monitoring |
| Potential Contamination Sources | DEP Databases and Water District |
| Land Use and Zoning | Water District and Municipal Records |
| Other Analyses | Other DEP Databases |

3.2  Source Sensitivity to Land Use

The key watershed features that define the sensitivity of the water supply source to land uses are briefly described below:

- Natural Features:   The watershed is largely forested and located in a remote area where public use and activities occur primarily in the summer and fall seasons.  The natural, undeveloped watershed conditions are protective of the water supply source.

- Man-made Features:   The western half of the pond shore is heavily developed primarily with seasonal camps and homes.   Roadways and access roads (mostly gravel) occur at a low density.   The eastern half of the pond shore is undeveloped.  The absence of controls on man-made activities and future development may diminish the buffer currently provided by the natural environment.

- Specific Activities:   Access to the pond and recreational activities in and around the pond are unrestricted.   There is a need for public awareness and management of recreational activities particularly in the eastern portion of the pond where the intake is located.

4.0   ASSESSMENT OF HANCOCK POND

Hancock Pond is used as a filtered and chlorinated water supply source.   Historically, the water quality has been good.  The Water Districts conduct annual testing (i.e., TE2 & TE3) of the raw water.  In 2000 and the first quarter of 2001, test results maintained by the Water Districts show that all parameters are within state and federal standards for public drinking water.  Daily testing of the raw water for pH and turbidity show values that range from 6.5 to 6.8 for pH and 0.4 to 0.5 NTU for turbidity.  Turbidity spikes can occur when there are heavy rains or when the pond water cycles (i.e., turns over) in the spring and fall.

4.1   Watershed

Hancock Pond and its watershed are of moderate size with only one small brook flowing into the pond on the northeast shore.  The depth of the pond varies significantly between the deeper western half and the shallow eastern half, i.e., a maximum of 105 feet compared to a maximum of 16 feet, respectively.  The upland watershed area is mostly undeveloped and in forest growth.  A low density of residential homes is present to the north of the pond and new residential  development is planned on the upland watershed area located to the west.  Minor farming activity occurs in the watershed.  Paper company land ownership includes a large portion of the upland watershed area located in Lexington TWP.  The land is managed for periodic timber harvesting.

In several locations, the upland slopes are steep and drain toward Hancock Pond.  In the event of a significant loss of trees due to forest fire, the slopes would become exposed to rainfall and increased runoff.   The added runoff could potentially increase the transport of eroded soils, dissolved solids and constituents derived from the burning of organic material.  Under these conditions, the pond would be more vulnerable to drainage in the burned portion of the watershed.

The water supply is located in Embden, New Portland and Lexington TWP, and therefore outside the municipal boundaries and daily supervision of the Madison and Anson Water Districts.  The Water Districts control the dam in order to manage the natural flow at the outlet of the dam.  The Water Districts have a right-of-way access across private property to reach the dam and conduct maintenance of the dam and intake screen.  Public access to the dam is subject to the controls, if any, of the private landowners.  The trails and roads in the watershed are used for off-road recreation, e.g., four-wheelers, snowmobiles, etc.

Review of the DEP databases of regulated activities and observations made during our  reconnaissance of the watershed found no significant point or non-point sources of potential contamination that would be a concern to the water supply.

4.2   Shoreland

The pond is divided into a heavily developed shoreland in the western half and mostly undeveloped shoreland in the eastern half.  The deeper and larger volume of the western half serves as a buffer (i.e., through dilution capacity) to man-made inputs from development.  In comparison, the shallow eastern half, where the intake is located, has less buffering capacity.  The shoreland is protected, in part, by zoning that extends 250 feet from the shore.  In addition, the no building provision of the Resource Protection Zone applies to most of the undeveloped shoreland in the eastern portion of the pond.

The Town of Embden has an active program to upgrade, replace or install new subsurface wastewater disposal systems for the private camps and seasonal homes located in the shoreland area.   Some camps continue to use outhouses appropriately setback from the pond although the town requires that “gray water” be properly disposed into a certified subsurface disposal system.

Most of the shoreland topography around the pond varies from gentle to moderate slopes.  Steeper slopes occur along the southeast shore in the vicinity (east and west) of the pond outlet.   The shoreland soils that are sensitive to erosion are shown in Figure 3.  If the forest is removed or damaged by fire, natural cover would be lost and soils would be exposed to increased runoff.   The Water Districts are aware of no critical erosion problems around the shore of the pond.

Historically, on the western portion of the pond, shore encroachment occurred as camps and seasonal homes became established.  This has resulted in disturbance along shorefront properties and the need to use rip-rap construction to maintain the banks along the shore.  Gravel roadways are present for access to these camps and are subject to erosion if not properly maintained.

The shoreland of the eastern half of the pond is mostly undeveloped and protected by zoning.

4.3   Intake

The water supply intake is located in the eastern portion of the pond approximately 240 feet from shore.  The intake is not identified by a buoy although the Water Districts have installed a sign on the shore near the pond outlet.   However, the intake location is in open water where recreational activity is currently unrestricted.   The summer residents enjoy boating, fishing, water skiing and other activities throughout the pond.  It was reported that the state has an interest to install a new public boat launch on the north shore in the eastern portion of the pond.   If this were to occur, increased recreational activities in this portion of the pond would raise the potential threat posed to the water supply.

Lakes and ponds normally experience an aging process known as eutrophication, which is caused by various natural and man-made influences.   Phosphorus from runoff, fertilizers, sewage and other sources is a primary factor affecting pond eutrophication since phosphorus nourishes plant and algae growth in the pond.   The amount of algae productivity in the pond is characterized in terms of three Trophic States: (1) Oligotrophic = low, (2) Mesotrophic = moderate, and (3) Eutrophic = high.  The Trophic State can be identified based on chlorophyll and phosphorus content and by secchi disk transparency data, which measures the clarity of the pond water.

The raw water from Hancock Pond is being monitored by the Water Districts.  Recent testing shows that turbidity is generally low and below 1 NTU.   Color was reported by the analytical laboratory in March 2001 at a value of 11.  Historical data in the PEARL database indicate secchi disk measurements at a depth of up to 22 feet below water surface.

According to a surface water classification database developed by the DEP, the Trophic State of Hancock Pond is listed as Mesotrophic.  The pond water is ranked as fully attaining the GPA classification with respect to aquatic life, primary contact (i.e., swimming), and trophic stability criteria.

Based on all the testing completed to date on the raw water from Hancock Pond, the pond is a good quality source.

5.0   SWAP RANKING AND RECOMMENDATIONS

5.1   Ranking of Susceptibility

The SWAP assessment factors indicate that overall susceptibility of the water quality in Hancock Pond falls between low to moderate.  The remoteness of the watershed, largely undeveloped and forested condition, zoning protection and management controls currently in place all support a low ranking.  However, the pond is susceptible for the following reasons: (1) half of the shoreland area is significantly developed with camps and seasonal homes, (2) recreational use on the pond is unrestricted and may increase in the future, (3) current and future water use may exceed safe yield, and (4) the Water Districts have minimal control since the source is located outside the jurisdiction of their own municipal boundaries.   The factors considered in assessing the overall risk are summarized in Table 3.

# TABLE  3

# HANCOCK POND SURFACE WATER ASSESSMENT

| Zone | Measure | Findings | Risk Level |
| --- | --- | --- | --- |
| Watershed | Ambient Water Quality Existing Conditions Future Development (Safe Yield) Overall | Class GPA, in full compliance. Land use characteristics present a low threat to water quality. Forest managed for timber harvesting, recreational vehicles in use on roads and trails, and residential development is sparse. No watershed protection controls exist. Pressure for future development appears low due to the remote location of the source. Evaluate water use vs. capacity of the source to meet current/future demand. | Low Low Low Low-Moderate Low |
| Shoreland | Pond Classification Soils Activities Posing a Threat Potential for Future Threats Overall | Mesotrophic Some potentially erodible soils along the shore and camp roads, (Lyman and Peru 8-15%), Berkshire (15-35%). Septic systems (active program to upgrade), gravel roads, encroachment from existing development. Development of private shoreland; possible interest in a new public boat launch site. | Low Low Low-Moderate Low-Moderate Low-Moderate |
| Intake | Raw Water Quality Ownership/Control Activities Posing a Threat Potential for Future Threats (Safe Yield) Overall | District testing shows good quality. Limited control at intake and source is outside District jurisdiction. Public access to intake area unrestricted. Increased recreational use on pond with no restrictions. Use of water in excess of safe yield reduces the source buffer capacity. | Low Low-Moderate Low-Moderate Low-Moderate Low -Moderate Low-Moderate |
| Overall |  |  | Low-Moderate |

5.2   Recommendations

The overall ranking for the pond susceptibility to threats of contamination is low to moderate.  Several additional actions could be considered by the Water Districts and the Town of Embden to provide added protection to the quality of the water supply.

- Re-evaluate safe yield, current and future use to determine if there are any limiting conditions that should be incorporated into the Water Districts’ long-term plans and short-term withdrawal strategy.

- The access roads into the dam and intake area are difficult to traverse with a vehicle.  However, the Water Districts should consider developing access controls (e.g., signs, gates, etc.) with the private landowners.

- Protection of the source intake is needed due to the sensitivity of the resource to recreational activities or other public access.  The posting of additional water supply signs along the shore of the pond should be considered to establish increased visibility and provide public awareness for the source.   Buoys and other forms of identification are suggested to direct recreational traffic away from the intake area particularly if a new public boat launch site is constructed nearby on the north shore.

- Develop and maintain a good working relationship with the paper company and other private landowners in the watershed that are managing the forest for wood production.  This is necessary in order to encourage forest cutting practices that are protective of the water resource, to avoid the use of pesticides and to minimize potential impacts from gravel roadways and logging roads.

- Shoreland areas currently have some protection under existing zoning provisions.  Further enhancement of this protection should be pursued through additional land purchases, conservation easements, recreation management tools, etc., and by working with Embden to encourage protective strategies for the source and watershed area.  The town’s program to upgrade septic systems is a good example of protecting the resource and raising public awareness.

APPENDIX A

##### SWAP ASSESSMENT GUIDELINES

MAINE PUBLIC DRINKING WATER

SOURCE WATER ASSESSMENT PROGRAM

MADISON AND ANSON WATER DISTRICTS

HANCOCK POND WATERSHED

# APPENDIX A

# ASSESSMENT GUIDELINES

| Assessment Item/Threat | Low | Moderate | Significant |
| --- | --- | --- | --- |
| Physical Characteristics of the Watershed Watershed boundary, area Area of water body Tributaries Watershed topography Wetlands Water body depth Sand and gravel aquifers Soil types (erodibility and slope) | smaller smaller few low many deep few low | >> >> >> moderate >> >> some moderate | larger larger many steep, rugged few shallow many high |
| Raw Water Quality Secchi Disk Transparency Turbidity pH Phosphorus Pesticides and herbicides Color Total Coliform and E Coli Dissolved Oxygen Temperature VOCs Chlorophyll-a Total Organic Carbon | > 8 M < 1 NTU 6.5-8.5 < 10 ppb ND < 5 > 29/100 ml >7 mg/L ambient absent < 2 ppb < 4 mg/L | 4-8 M < 5 NTU N/A < 20 ppb N/A 5-15 occasional 5-7 mg/L N/A 2-6 ppb 4-8 mg/L | < 4 M > 5 NTU < 6.5 or > 8.5 > 30 ppb detected > 15 > 142/100 ml < 5 mg/L > 10o C ambient detected > 6 ppb > 8 mg/L |
| Potential Contamination Source Inventory UST/AST leak sites Municipal/Comm/Special Landfill Compost sites Demolition debris sites Ash/septage storage/utilization Superfund site Sand/salt storage sites Petroleum spill sites Hazardous waste sites Wastewater treatment facilities Food residuals utilization sites Uncontrolled hazardous sites Tank farms Industrial complexes | Absent from watershed or actively monitored and controlled or fully remediated. | Present in watershed, some operational deficiencies and/or remediation is underway. | Present and unmonitored and/or unremediated. |

| Assessment Item/Threat | Low | Moderate | Significant |
| --- | --- | --- | --- |
| Potential Contamination Source Inventory Non-point pollution sources Transfer stations Sludge utilization sites Automobile graveyards Engineered subsurface systems Woodyards Underground injection wells Surface impoundments Mining/mineral processing sites Overboard discharges to rivers Active UST sites Roadways Railroads Utility Rights-of-way Boat launches Commercial farms CAFOs Commercial forest operations Private septic systems Home heating oil tanks Airports Bathing beaches Combined sewage overflows | If present, at low density, properly installed and operated. In compliance with applicable local, state and federal regulations. | >>>> | High density, little evidence of active management, documented incidence of problems and failures. |
| Land Uses and Zoning Residential density Percent of watershed Owned by PWS Public lands or conservation Forested With protective zoning Agricultural Impervious cover Industrial/commercial Waterbody uses: Restricted areas Ice fishing Boating Swimming Recreational vehicles | > 2 Ac/lot total > 80% for all protected uses < 20 % total < 20 % total < 20 % total large, posted limited to absent | 1-2 Ac/lot intake and shoreland protected, others developed 20-30% total 20-30% total 20-30% total intake zone moderate, well-managed | < 1 Ac/lot critical areas (intake, shoreland) unprotected or developed > 30% total of 3 > 30% total of 3 > 30% total of 3 small to none prevalent, uncontrolled |
| Other Analyses Trophic State Index Vulnerability Index Build-out Analysis | < 25 low fully developed | 25-60 moderate >>> | > 60 high high potential for development |